## Circuit Diagram: -

## Code

## #include <Arduino.h>

#include <WiFi.h>

#include <AsyncTCP.h>

#include <ESPAsyncWebServer.h>

#include <ESP32Servo.h>

#include <iostream>

#include <sstream>

struct ServoPins

{

Servo servo;

int servoPin;

String servoName;

int initialPosition;

};

std::vector<ServoPins> servoPins =

{

{ Servo(), 27 , "Base", 90},

{ Servo(), 26 , "Shoulder", 90},

{ Servo(), 25 , "Elbow", 90},

{ Servo(), 33 , "Gripper", 90},

};

struct RecordedStep

{

int servoIndex;

int value;

int delayInStep;

};

std::vector<RecordedStep> recordedSteps;

bool recordSteps = false;

bool playRecordedSteps = false;

unsigned long previousTimeInMilli = millis();

const char* ssid     = "RobotArm";

const char* password = "12345678";

AsyncWebServer server(80);

AsyncWebSocket wsRobotArmInput("/RobotArmInput");

const char* htmlHomePage PROGMEM = R"HTMLHOMEPAGE(

<!DOCTYPE html>

<html>

<head>

<meta name="viewport" content="width=device-width, initial-scale=1, maximum-scale=1, user-scalable=no">

<style>

input[type=button]

{

background-color:red;color:white;border-radius:30px;width:100%;height:40px;font-size:20px;text-align:center;

}

.noselect {

-webkit-touch-callout: none; /* iOS Safari */

-webkit-user-select: none; /* Safari */

-khtml-user-select: none; /* Konqueror HTML */

-moz-user-select: none; /* Firefox */

-ms-user-select: none; /* Internet Explorer/Edge */

user-select: none; /* Non-prefixed version, currently supported by Chrome and Opera */

}

.slidecontainer {

width: 100%;

}

.slider {

-webkit-appearance: none;

width: 100%;

height: 20px;

border-radius: 5px;

background: #d3d3d3;

outline: none;

opacity: 0.7;

-webkit-transition: .2s;

transition: opacity .2s;

}

.slider:hover {

opacity: 1;

}

.slider::-webkit-slider-thumb {

-webkit-appearance: none;

appearance: none;

width: 40px;

height: 40px;

border-radius: 50%;

background: red;

cursor: pointer;

}

.slider::-moz-range-thumb {

width: 40px;

height: 40px;

border-radius: 50%;

background: red;

cursor: pointer;

}

</style>

</head>

<body class="noselect" align="center" style="background-color:white">

<h2 style="color: teal;text-align:center;">Robot Arm Control</h2>

<table id="mainTable" style="width:400px;margin:auto;table-layout:fixed" CELLSPACING=10>

<tr/><tr/>

<tr/><tr/>

<tr>

<td style="text-align:left;font-size:25px"><b>Gripper:</b></td>

<td colspan=2>

<div class="slidecontainer">

<input type="range" min="0" max="180" value="90" class="slider" id="Gripper" oninput='sendButtonInput("Gripper",value)'>

</div>

</td>

</tr>

<tr/><tr/>

<tr>

<td style="text-align:left;font-size:25px"><b>Elbow:</b></td>

<td colspan=2>

<div class="slidecontainer">

<input type="range" min="0" max="180" value="90" class="slider" id="Elbow" oninput='sendButtonInput("Elbow",value)'>

</div>

</td>

</tr>

<tr/><tr/>

<tr>

<td style="text-align:left;font-size:25px"><b>Shoulder:</b></td>

<td colspan=2>

<div class="slidecontainer">

<input type="range" min="0" max="180" value="90" class="slider" id="Shoulder" oninput='sendButtonInput("Shoulder",value)'>

</div>

</td>

</tr>

<tr/><tr/>

<tr>

<td style="text-align:left;font-size:25px"><b>Base:</b></td>

<td colspan=2>

<div class="slidecontainer">

<input type="range" min="0" max="180" value="90" class="slider" id="Base" oninput='sendButtonInput("Base",value)'>

</div>

</td>

</tr>

<tr/><tr/>

<tr>

<td style="text-align:left;font-size:25px"><b>Record:</b></td>

<td><input type="button" id="Record" value="OFF" ontouchend='onclickButton(this)'></td>

<td></td>

</tr>

<tr/><tr/>

<tr>

<td style="text-align:left;font-size:25px"><b>Play:</b></td>

<td><input type="button" id="Play" value="OFF" ontouchend='onclickButton(this)'></td>

<td></td>

</tr>

</table>

<script>

var webSocketRobotArmInputUrl = "ws:\/\/" + window.location.hostname + "/RobotArmInput";

var websocketRobotArmInput;

function initRobotArmInputWebSocket()

{

websocketRobotArmInput = new WebSocket(webSocketRobotArmInputUrl);

websocketRobotArmInput.onopen    = function(event) {};

websocketRobotArmInput.onclose   = function(event) { setTimeout(initRobotArmInputWebSocket, 2000); };

websocketRobotArmInput.onmessage    = function(event)

{

var keyValue = event.data.split(",");

var button = document.getElementById(keyValue[0]);

button.value = keyValue[1];

if (button.id == "Record" || button.id == "Play")

{

button.style.backgroundColor = (button.value == "ON" ? "green" : "red");

enableDisableButtonsSliders(button);

}

};

}

function sendButtonInput(key, value)

{

var data = key + "," + value;

websocketRobotArmInput.send(data);

}

function onclickButton(button)

{

button.value = (button.value == "ON") ? "OFF" : "ON" ;

button.style.backgroundColor = (button.value == "ON" ? "green" : "red");

var value = (button.value == "ON") ? 1 : 0 ;

sendButtonInput(button.id, value);

enableDisableButtonsSliders(button);

}

function enableDisableButtonsSliders(button)

{

if(button.id == "Play")

{

var disabled = "auto";

if (button.value == "ON")

{

disabled = "none";

}

document.getElementById("Gripper").style.pointerEvents = disabled;

document.getElementById("Elbow").style.pointerEvents = disabled;

document.getElementById("Shoulder").style.pointerEvents = disabled;

document.getElementById("Base").style.pointerEvents = disabled;

document.getElementById("Record").style.pointerEvents = disabled;

}

if(button.id == "Record")

{

var disabled = "auto";

if (button.value == "ON")

{

disabled = "none";

}

document.getElementById("Play").style.pointerEvents = disabled;

}

}

window.onload = initRobotArmInputWebSocket;

document.getElementById("mainTable").addEventListener("touchend", function(event){

event.preventDefault()

});

</script>

</body>

</html>

)HTMLHOMEPAGE";

void handleRoot(AsyncWebServerRequest *request)

{

request->send_P(200, "text/html", htmlHomePage);

}

void handleNotFound(AsyncWebServerRequest *request)

{

request->send(404, "text/plain", "File Not Found");

}

void onRobotArmInputWebSocketEvent(AsyncWebSocket *server,

AsyncWebSocketClient *client,

AwsEventType type,

void *arg,

uint8_t *data,

size_t len)

{

switch (type)

{

case WS_EVT_CONNECT:

Serial.printf("WebSocket client #%u connected from %s\n", client->id(), client->remoteIP().toString().c_str());

sendCurrentRobotArmState();

break;

case WS_EVT_DISCONNECT:

Serial.printf("WebSocket client #%u disconnected\n", client->id());

break;

case WS_EVT_DATA:

AwsFrameInfo *info;

info = (AwsFrameInfo*)arg;

if (info->final && info->index == 0 && info->len == len && info->opcode == WS_TEXT)

{

std::string myData = "";

myData.assign((char *)data, len);

std::istringstream ss(myData);

std::string key, value;

std::getline(ss, key, ',');

std::getline(ss, value, ',');

Serial.printf("Key [%s] Value[%s]\n", key.c_str(), value.c_str());

int valueInt = atoi(value.c_str());

if (key == "Record")

{

recordSteps = valueInt;

if (recordSteps)

{

recordedSteps.clear();

previousTimeInMilli = millis();

}

}

else if (key == "Play")

{

playRecordedSteps = valueInt;

}

else if (key == "Base")

{

writeServoValues(0, valueInt);

}

else if (key == "Shoulder")

{

writeServoValues(1, valueInt);

}

else if (key == "Elbow")

{

writeServoValues(2, valueInt);

}

else if (key == "Gripper")

{

writeServoValues(3, valueInt);

}

}

break;

case WS_EVT_PONG:

case WS_EVT_ERROR:

break;

default:

break;

}

}

void sendCurrentRobotArmState()

{

for (int i = 0; i < servoPins.size(); i++)

{

wsRobotArmInput.textAll(servoPins[i].servoName + "," + servoPins[i].servo.read());

}

wsRobotArmInput.textAll(String("Record,") + (recordSteps ? "ON" : "OFF"));

wsRobotArmInput.textAll(String("Play,") + (playRecordedSteps ? "ON" : "OFF"));

}

void writeServoValues(int servoIndex, int value)

{

if (recordSteps)

{

RecordedStep recordedStep;

if (recordedSteps.size() == 0) // We will first record initial position of all servos.

{

for (int i = 0; i < servoPins.size(); i++)

{

recordedStep.servoIndex = i;

recordedStep.value = servoPins[i].servo.read();

recordedStep.delayInStep = 0;

recordedSteps.push_back(recordedStep);

}

}

unsigned long currentTime = millis();

recordedStep.servoIndex = servoIndex;

recordedStep.value = value;

recordedStep.delayInStep = currentTime - previousTimeInMilli;

recordedSteps.push_back(recordedStep);

previousTimeInMilli = currentTime;

}

servoPins[servoIndex].servo.write(value);

}

void playRecordedRobotArmSteps()

{

if (recordedSteps.size() == 0)

{

return;

}

// This is to move servo to initial position slowly. First 4 steps are initial position

for (int i = 0; i < 4 && playRecordedSteps; i++)

{

RecordedStep &recordedStep = recordedSteps[i];

int currentServoPosition = servoPins[recordedStep.servoIndex].servo.read();

while (currentServoPosition != recordedStep.value && playRecordedSteps)

{

currentServoPosition = (currentServoPosition > recordedStep.value ? currentServoPosition - 1 : currentServoPosition + 1);

servoPins[recordedStep.servoIndex].servo.write(currentServoPosition);

wsRobotArmInput.textAll(servoPins[recordedStep.servoIndex].servoName + "," + currentServoPosition);

delay(50);

}

}

delay(2000); // Delay before starting the actual steps.

for (int i = 4; i < recordedSteps.size() && playRecordedSteps ; i++)

{

RecordedStep &recordedStep = recordedSteps[i];

delay(recordedStep.delayInStep);

servoPins[recordedStep.servoIndex].servo.write(recordedStep.value);

wsRobotArmInput.textAll(servoPins[recordedStep.servoIndex].servoName + "," + recordedStep.value);

}

}

void setUpPinModes()

{

for (int i = 0; i < servoPins.size(); i++)

{

servoPins[i].servo.attach(servoPins[i].servoPin);

servoPins[i].servo.write(servoPins[i].initialPosition);

}

}

void setup(void)

{

setUpPinModes();

Serial.begin(115200);

WiFi.softAP(ssid, password);

IPAddress IP = WiFi.softAPIP();

Serial.print("AP IP address: ");

Serial.println(IP);

server.on("/", HTTP_GET, handleRoot);

server.onNotFound(handleNotFound);

wsRobotArmInput.onEvent(onRobotArmInputWebSocketEvent);

server.addHandler(&wsRobotArmInput);

server.begin();

Serial.println("HTTP server started");

}

void loop()

{

wsRobotArmInput.cleanupClients();

if (playRecordedSteps)

{

playRecordedRobotArmSteps();

}

}